SECTION 09 68 16 SHEET CARPETING

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes the following:

          - Sheet carpet.

        - Related Requirements:

RETAIN SUBPARAGRAPHS BELOW TO CROSS-REFERENCE REQUIREMENTS CONTRACTOR MIGHT EXPECT TO FIND IN THIS SECTION BUT ARE SPECIFIED IN OTHER SECTIONS.

          - Section 02 41 19 "Selective Demolition" for removing existing floor coverings.

          - Section 09 05 65 “Preinstallation Testing for Floor Finishes” for substrate moisture and alkalinity tests to be performed on concrete substrates.

          - Section 09 65 13 "Resilient Base and Accessories" for resilient wall base and accessories installed with carpet.

          - Section 09 68 13 "Tile Carpeting" for modular carpet tiles.

      - PREINSTALLATION MEETINGS

RETAIN "PREINSTALLATION CONFERENCE" PARAGRAPH BELOW IF WORK OF THIS SECTION IS EXTENSIVE OR COMPLEX ENOUGH TO JUSTIFY A CONFERENCE.

        - Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

RETAIN SUBPARAGRAPH BELOW IF ADDITIONAL REQUIREMENTS ARE NECESSARY.

          - Review methods and procedures related to carpet installation including, but not limited to, the following:

RETAIN FIRST SUBPARAGRAPH FOR OCCUPIED BUILDINGS IF DELIVERY MUST BE CONTROLLED AND COORDINATED, FOR EXAMPLE, FOR TENANT SPACES IN OCCUPIED OFFICE BUILDINGS WITH MULTIPLE TENANTS.

            - Review delivery, storage, and handling procedures.

            - Review ambient conditions and ventilation procedures.

            - Review subfloor preparation procedures.

            - <Insert agenda items>.

      - SUBMITTALS

        - Product Data:  For each type of product indicated.  Include manufacturer's written data on physical characteristics, durability, and fade resistance. Include installation recommendations for each type of substrate required.

        - Shop Drawings:  Show the following:

          - Columns, doorways, enclosing walls or partitions, built-in cabinets, and locations where cutouts are required in carpet.

RETAIN THE TWO FOLLOWING SUBPARAGRAPHS FOR EXISTING CONDITIONS.

          - Existing flooring materials to be removed.

          - Existing flooring materials to remain.

          - Carpet type, color, and dye lot.

          - Seam locations, types, and methods.

          - Type of sub-floor.

          - Type of installation.

          - Pattern type, repeat size, location, direction, and starting point.

          - Pile direction.

          - Type, color, and location of insets and borders.

          - Type, color, and location of edge, transition, and other accessory strips.

          - Transition details to other flooring materials.

        - Samples:  For each of the following products and for each color and texture required.  Label each Sample with manufacturer's name, material description, color, pattern, and designation indicated on Drawings and in schedules.

          - Carpet:  12-inch-(300-mm-) square Sample.

          - Exposed Edge Stripping and Accessory:  12-inch-(300-mm-) long Samples.

          - Carpet Seam:  6-inch (150-mm) Sample.

          - Mitered Carpet Border Seam:  12-inch-(300-mm-) square Sample.  Show carpet pattern alignment.

        - Product Schedule:  Use same designations indicated on Drawings.

COORDINATE "QUALIFICATION DATA" PARAGRAPH BELOW WITH QUALIFICATION REQUIREMENTS IN SECTION 014000 "QUALITY REQUIREMENTS" AND AS MAY BE SUPPLEMENTED IN "QUALITY ASSURANCE" ARTICLE.

        - Qualification Data: For Installer.

        - Product Test Reports: For carpet[ and carpet cushion], for tests performed by a qualified testing agency.

        - Sample Warranties: For special warranties.

        - Maintenance Data:  For carpet to include in maintenance manuals specified in Division 01.  Include the following:

          - Methods for maintaining carpet, including cleaning and stain-removal products and procedures and manufacturer's recommended maintenance schedule.

          - Precautions for cleaning materials and methods that could be detrimental to carpet.

        - Record Documents:  Showing locations of substrate moisture and alkalinity tests.  Provide markups on floor plan indicating the location of each test and the dates tests were performed.

      - QUALITY ASSURANCE

        - Installer Qualifications:  An experienced installer who is certified by the Floor Covering Installation Contractors Association or who can demonstrate compliance with its certification program requirements.

        - Product Options:  Products and manufacturers named in Part 2 establish requirements for product quality in terms of appearance, construction, and performance.  Other manufacturers' products comparable in quality to named products and complying with requirements may be considered.  Refer to Division 01 Section "Common Product Requirements."

        - Fire-Test-Response Characteristics:  Provide products with the critical radiant flux classification indicated in Part 2, as determined by testing identical products per NFPA 253 by an independent testing and inspecting agency acceptable to authorities having jurisdiction.

        - Preinstallation Conference:  Conduct conference at Project site to comply with requirements in Division 01 Section "Project Management and Coordination."

          - Conduct conference with attendance and participation of the following:

            - Owner.

            - Architect.

            - Interior Designer.

            - Contractor.

            - Flooring Installer.

            - Flooring manufacturer's representative.

            - Testing agency representative.

          - Review methods and procedures related to flooring installation, including manufacturer's written instructions.

          - Review concrete substrate requirements for conditions affecting performance of flooring, including results of moisture and alkalinity tests.

          - Review locations and frequency of moisture and alkalinity tests.

        - Mockups:  Before installing carpet, install mockups for each type of carpet installation required to demonstrate aesthetic effects and qualities of materials and execution.  Install mockups to comply with the following requirements, using materials indicated for the completed Work:

          - Install mockups in the location and of the size indicated or, if not indicated, as directed by Architect.

          - Notify Architect seven days in advance of dates and times when mockups will be installed.

          - Demonstrate the proposed range of aesthetic effects and workmanship.

          - Obtain Architect's approval of mockups before starting work.

          - Maintain mockups during construction in an undisturbed condition as a standard for judging the completed Work.

          - Remove unacceptable mockups when directed.

          - Approved mockups may become part of the completed Work if undamaged at time of Substantial Completion.

      - DELIVERY, STORAGE, AND HANDLING

        - General:  Comply with CRI 104, Section 5, "Storage and Handling."

        - Deliver carpet in original mill protective covering with mill register numbers and tags attached.

      - PROJECT CONDITIONS

        - General:  Comply with CRI 104, Section 6.1, "Site Conditions; Temperature and Humidity."

        - Environmental Limitations:  Do not install carpet until building is enclosed, wet work is complete, and HVAC system is operating and maintaining temperature and relative humidity at occupancy levels during the remainder of the construction period.

        - Do not install carpet over concrete slabs until slabs have cured and are sufficiently dry to bond with adhesive, as determined by floor covering manufacturer's recommendations, and field moisture and alkalinity tests.

        - Where demountable partitions or other items are indicated for installation on top of carpet, install carpet before installing these items.

      - WARRANTY

WHEN WARRANTIES ARE REQUIRED, VERIFY WITH OWNER'S COUNSEL THAT WARRANTIES STATED IN THIS ARTICLE ARE NOT LESS THAN REMEDIES AVAILABLE TO OWNER UNDER PREVAILING LOCAL LAWS.

        - General Warranty:  The special warranty specified in this Section shall not deprive the Owner of other rights the Owner may have under other provisions of the Contract Documents and shall be in addition to, and run concurrent with, other warranties made by the Contractor under requirements of the Contract Documents.

[REVISE WARRANTY TO SUIT PROJECT.  VERIFY AVAILABLE WARRANTIES FROM MFR.]

        - Special Carpet Warranty:  Submit a written warranty executed by carpet manufacturer and Installer agreeing to repair or replace carpet that does not meet requirements or that fails in materials or workmanship within the specified warranty period.

          - Manufacturer’s Warranty: Provide a non-prorated lifetime limited warranty from date of Substantial Completion for failures which include, but are not limited to:

            - Wear: Carpet materials shall evidence no more than 15 percent loss of pile fiber weight.

            - Edge Raveling: Carpet material shall exhibit no edge ravel, snags, and runs under normal use.

            - Delamination: Carpet materials shall not delaminate from primary or secondary backing.

            - Shrinkage:  Carpet materials shall show no shrinkage.

            - Resiliency: Backing shall show no loss of resiliency.

            - Excessive Static Electricity: More than 3.5 kV per AATCC 134.

          - Provide a manufacturer’s yarn warranty of ten (10) years from date of Substantial Completion for colorfastness.  Carpet shall not fade due to exposure to light or harsh cleaning chemicals.

          - Provide a manufacturer’s lifetime yarn warranty against staining and failure of tuft bind.

          - Installation Warranty: Written warranty, signed by carpet installer and general contractor agreeing to correct faulty workmanship for a period of 2 years from date of Substantial Completion.

- PRODUCTS

      - CARPET

        - Acceptable Manufacturers:

          - Atlas.

          - Interface, LLC.

          - Mannington Mills, Inc.

          - Mohawk Group (The); Mohawk Carpet, LLC.

          - Tandus; a Tarkett company.

        - Products:  Refer to Interior Finish Legend on Drawings.

EDIT BELOW TO MATCH PRODUCT SELECTED OR KEEP ABOVE TO LIST BASIS OF DESIGN PRODUCT.

        - Color: [As indicated by manufacturer's designations] [Match Architect's samples] [As selected by Architect from manufacturer's full range].

          - Color: Mixture of multiple dark and light tones. Monochromatic solid or light colors are unacceptable.

        - Pattern: [Match Architect's samples] <Insert pattern>.

        - Construction: Tufted, level or multi-level loop pile with maximum pile height variation of 1/32 inch.

          - Tuft bind: ASTM D1335 – must resist a minimum of 10.0 lbs. force.

        - Yarn: 100% first quality, Type 6,6 bulk continuous filament (BCF) nylon. Hollow filament fiber shapes for optimum soil hiding capacity.

          - Polymer type identification to AATCC 20.

          - Antron Legacy or Lumena, or approved equal.

          - Verification test of fiber shall be completed by manufacturer if a custom pattern is created.

        - Dye Method: 100% solution dyed or 100% yarn dyed, or combination of both, subject to compliance with requirements.

        - Primary Backing: [Manufacturer's standard material] [Woven polypropylene] [Nonwoven, polypropylene or polyester] <Insert specific primary backing material>.

        - Secondary Backing: [Manufacturer's standard material] [Woven polypropylene] [Nonwoven, polypropylene or polyester] [Woven jute] [Fiberglass] <Insert specific secondary backing material>.

        - Roll Width: [12 feet (3.7 m)] [6 feet (1.8 m)] [13.5 feet (4.1 m)] [15 feet (4.6 m)] <Insert dimension>.

        - Physical Performance Requirements:

[if additional information is needed for the item below, refer to the Carpet and Rug Institute guidelines for TARR].

          - Texture Retention Rating: Vettermann Drum test method ASTM D5417 for 22,000 cycles with a minimum rating of 3.0 or Hexapod test method ASTM D5252 for 12,000 cycles (8.4 lb. tumbler) with a minimum rating of 3.5.

            - Obtain rating using the appropriate Commercial Reference Scale for the construction, per ASTM D7330.

            - Conduct testing without underpad or brushing.

          - Pile Density: Minimum 5500, for heavy or severe traffic. Pile density = 36 x pile weight (oz./sq. yd.) / pile thickness (inches). Pile thickness: ASTM D6859 or ASTM D7241.

          - Stain Resistance: AATCC 171 for two washings to simulate removal of topical treatments by hot water extractions, followed by AATCC 175.

            - Spills Likely: Minimum rating of 8 using AATCC Red 40 Stain Scale.

            - Spills a Concern: Minimum rating of 6 using AATCC Red 40 Stain Scale.

          - Soil Resistance: Treatment shall be heat- applied by the mill.

            - Hollow filament fiber shape for optimum soil hiding and soil release capability.

            - DuraTech® soil resistance treatment or equal.

          - Colorfastness to Light: AATCC 16 part 3, 200 AFU, AATCC Gray Scale for Color Change rating minimum of 3-4.

          - Colorfastness to Crocking: AATCC 165, minimum rating of 4 on AATCC Chromatic Transference Scale.

          - Colorfastness to Atmospheric Contaminants: AATCC 164 and AATCC 129 for 2 cycles; minimum rating of 3-4 using AATCC Gray Scale for Color Change.

          - Static Control: Achieved by permanent means (i.e. antistatic filaments) and without chemical treatment.

            - AATCC 134, to include step and scuff tests.

              - Static generation below 3.5 kilovolts.

          - Resistance to Delamination: ASTM D3936, minimum 10.0 lbs./inch, no delamination.

          - Flammability:

            - Smoke Density: ASTM E662, rating of less than 450 Dm in flaming mode; or to comply with State Code.

            - Critical Radiant Flux: NFPA Class 1, with minimum of 0.45 watts/sq. cm, when tested in accordance with ASTM E648 or NFPA 253; or to comply with State Code.

            - Ignition Characteristics: Pass ASTM D2859 or CPSC FF1-70.

          - Dimensional Stability:  Aachen method/ ISO 2551. Maximum change plus/minus 0.20%.

        - Recycling:  Carpet shall be eligible for recycling by the supplying mill to an existing operational third party certified recycling center.

          - Carpet requiring use of landfills is unacceptable.

        - Sustainability:  Certified at the V4 Silver Level in accordance with NSF/ANSI 140.

      - MANUFACTURED UNITS

        - Carpet shall have a lead-time of not more than six (6) weeks from the time the dealer receives the order to the time delivered to Houston area warehouse.

        - Patterns and colorways shall be available for a minimum of ten (10) years.

        - Carpet within the same floor of a project shall be from the same dye lot.

      - INSTALLATION ACCESSORIES

        - Trowelable Leveling and Patching Compounds:  Latex-modified, hydraulic-cement-based formulation provided by or recommended by the following:

          - Carpet manufacturer.

        - Adhesives:  Water-resistant, mildew-resistant, non-staining type to suit products and sub-floor conditions indicated, that complies with flammability requirements for installed carpet and that is recommended by the following:

          - Carpet manufacturer.

- EXECUTION

      - EXAMINATION

        - Examine substrates, areas, and conditions for compliance with requirements for maximum moisture content, alkalinity range, installation tolerances, and other conditions affecting carpet performance.  Verify that substrates and conditions are satisfactory for carpet installation and comply with requirements specified.

        - Concrete Sub-floors:  Verify that concrete slabs comply with ASTM F 710 and the following:

          - Slab substrates are dry and free of curing compounds, sealers, hardeners, and other materials that may interfere with adhesive bond.  Determine adhesion and dryness characteristics by performing substrate moisture and alkalinity tests as specified in Division 09 Section “Preinstallation Testing for Floor Finishes.”

          - Sub-floor finishes comply with requirements specified in Division 03 Section “Structural Cast-in-Place Concrete” for slabs receiving carpet.

          - Sub-floors are free of cracks, ridges, depressions, scale, and foreign deposits.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREINSTALLATION TESTING

        - General:  Perform preinstallation testing under provisions of Section 09 05 65.

        - Submit floor covering and adhesive manufacturer’s written acceptance of the concrete floor substrate as represented by moisture and alkalinity testing.

        - Submit copies of test reports and flooring manufacturer’s written acceptance of substrate conditions to Owner and Architect prior to flooring installation.

        - If test results indicate concrete subfloor is not within flooring manufacturers’ acceptable range, notify Owner and Architect.

      - PREPARATION

        - General:  Comply with CRI 104, Section 6.2, "Site Conditions; Floor Preparation," and carpet manufacturer's written installation instructions for preparing substrates indicated to receive carpet installation.

        - Use trowelable leveling and patching compounds, according to manufacturer's written instructions, to fill cracks, holes, and depressions in substrates.

        - Remove coatings, including curing compounds, and other substances that are incompatible with adhesives and that contain soap, wax, oil, or silicone, without using solvents.  Use mechanical methods recommended in writing by the following:

          - Carpet manufacturer.

        - Broom and vacuum clean substrates to be covered immediately before installing carpet.  After cleaning, examine substrates for moisture, alkaline salts, carbonation, or dust.  Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Direct-Glue-Down Installation:  Comply with CRI 104, Section 8, "Direct Glue-Down Installation."

retain following subparagraph if required for project.  coordinate with owner’s rep

          - Seams:  Chemically weld.

        - Pre-Applied Adhesive:  Comply with manufacturer’s instructions.

        - Comply with carpet manufacturer's written recommendations for seam locations and direction of carpet; maintain uniformity of carpet direction and lay of pile.  At doorways, center seams under the door in closed position.

          - Bevel adjoining border edges at seams with hand shears.

        - Do not bridge building expansion joints with carpet.

        - Cut and fit carpet to butt tightly to vertical surfaces, permanent fixtures, and built-in furniture including cabinets, pipes, outlets, edgings, thresholds, and nosings.  Bind or seal cut edges as recommended by carpet manufacturer.

        - Extend carpet into toe spaces, door reveals, closets, open-bottomed obstructions, removable flanges, alcoves, and similar openings.

        - Maintain reference markers, holes, and openings that are in place or marked for future cutting by repeating on finish flooring as marked on sub-floor.  Use nonpermanent, non-staining marking device.

        - Install pattern parallel to walls and borders, maintaining continuity of pattern at seams.

[retain only for large projects]

      - SITE QUALITY CONTROL

        - The Owner reserves the right to take random samples from production runs and/or from delivered carpeting for testing by an independent  testing laboratory for abrasion resistance, soiling, crushing, breaking strength, distortion, elongations, loop strength, and color fastness, etc., in order to ascertain if manufactured and/or delivered materials conform to approved samples and specifications.

          - Provide 5 square yards of actual production pieces for each color for testing.

          - Failed testing of production sample shall be at the expense of the Contractor.

          - Carpeting which does not conform to approved samples and specifications will be rejected.

        - Manufacturer's Duties:

          - At the beginning of installation and a minimum of once during the installation and once upon completion of the installation, a representative of the carpet manufacturer shall inspect the installation to ascertain that the work is being installed according to the manufacturer's published recommendations, project specifications and manufacturer's warranty requirements.

            - Inspector shall be an employee of the manufacturer and not of the contractor, distributor, or manufacturer's agent.

          - Within 48 hours following each inspection, notify Owner, Contractor, and Architect in writing of conditions that would preclude issuance of manufacturer's warranty.

      - CLEANING

        - Perform the following operations immediately after installing carpet:

          - Remove excess adhesive, seam sealer, and other surface blemishes using cleaner recommended by carpet manufacturer.

          - Remove yarns that protrude from carpet surface.

          - Vacuum carpet using commercial machine with face-beater element.

      - DEMONSTRATION

        - Carpet manufacturer shall provide minimum of two hours, on site, in-service training for facility’s maintenance personnel.

      - PROTECTION

        - Protect installed carpet to comply with CRI 104, Section 15, "Protection of Indoor Installations."

        - Protect carpet against damage from construction operations and placement of equipment and fixtures during the remainder of construction period.  Use protection methods indicated or recommended in writing by carpet manufacturer.

END OF SECTION 09 68 16

FOR MDACC USE ONLY. DELETE THIS PAGE BEFORE ISSUING DOCUMENTS.

document revision history

| Issue | Date | Revision Description | Author |
| --- | --- | --- | --- |
|  | 20190301 | Original Issuance |  |
| Rev. 1 |  | . |  |
| Rev. 2 |  |  |  |
| Rev. 3 |  |  |  |
| Rev. 4 |  |  |  |
| Rev. 5 |  |  |  |